# Visual Inspection Checklist for Piping Systems

Note: Piping s ystems also include injection points, pumps, compress ors, valves, filters, tubing and tube fittings , connected mechanical equipment, instrumentation, and pipe support systems

- IDENTIFICATION

Circuit Unique ID Number:	Piping Line Number/Name:	Date:

Piping System Name:	Bldg:

Date of Inspection:	Work Order Number:		Pipe Specification #: Inspector:		Employee Number:		Phone: Comments:

- FIELD IDENTIFICATION	 Yes	No	Location / Comment

Is piping labeled/tagged w/Equipment ID#? Is pipeline adequately identified?

Are labels/tags readable?

Do labels/tags need replacing?

- LEAKS	 Yes	No	Location / Comment

Any leaks from the process?

Any leaks from steam tracing?

Are any valves leaking?

Any leaks at leak repair clamps?

Any evidence of past leaks?

- PIPE SUPPORT	 Yes	No	Location / Comment

Any shoes off their supports?

Any hangers missing or damaged?

Any spring hangers bottomed-out?

Any problems with support braces?

*Any corrosion of supports?

Any loose or broken brackets?

- VIBRATION	 Yes	No	Location / Comment

Any significant vibration observed?

Any evidence of excessive movement?

Any pipe distortion observed?

- INSULATION **	 Yes	No	Location / Comment

Any physical damage or penetration?

Any deterioration/damage of weather seals?

Any bulging or wet insulation?

Any discoloration indicating leakage?

Any retaining bands missing/broken?

Any missing jackets, plugs, or insulation?

- CORROSION *	Severe  Mild	No	Location / Comment

Any corrosion at support points or fixtures?

Any coating or paint  deterioration?

Any areas with scale, pits, or rust?

Any corrosion between flanges?

Any significant corrosion of f lange bolts?

Continued on next page

Revised	Page 1 of 2

Circuit Unique ID Number:

8. INSPECTOR

Were any Deficiencies Identified?

Piping Line Number/Name:

Date:

No

Yes, see Comments

Is Owner follow-up required to address issues that are concerns, but not deficiencies:

No

Yes, see Comments

Deficiencies and Issues of Concern have been Communicated to Equipment Owner or Delegate (e.g., Maintenance Coordinator)

N/A	Yes, Person Notified:	No, Explain in Comments

Deficiency

Categories:

Leaks

Pipe Supports

Vibration

Corrosion

Insulation

Other:

Qualified Person / Inspector:	Employee Number:	Date:

Owner (Signature required only if Deficiencies or Issues of Concern are identified):	Employee Number:	Date:

Comments:

Visual Inspection Checklist for Piping Systems

Note: Send completed form to the Maintenance Representative who updates GEMTS/Maintenance records and then sends the form to the Mechanical Integrity Equipment File Administrator.  This form shall be permanently filed in the Mechanical Integrity Equipment File.

* For assistance in determining degree of severity call NDE // ** For assistance in determining the extent of damage call Technical Resources

Revised:	Page 2 of 2